## CH332Lab 6:  FTIR AND RAMAN ANALYSIS OF ETHANOL IN VODKA

Pre-lab problem:  In this experiment, you will be making IR and Raman vibrational frequency measurements of ethanol in Vodka using the method of standard additions. To be sure that you understand the sample preparation and the calculations associated with this method, please use EXCEL to work through problem 5-27 (a and b) in HARRIS.  (Please save your excel work to use for your template to analyze the data collected in lab.  It will help you in the ACS write up!!!). Please hand in answers to all parts of this problem, including plots/spreadsheets/line stats etc. Both are due before you attempt to complete this laboratory exercise (4/10/2012).

WebMo calculations: IR and Raman vibrational frequencies can be generated using WebMo/Gaussian. (Firefox is preferred).  Open the WebMo applet from the Schupf lab home page (schupflab.colby.edu), the PCHEM homepage and choose the WebMo "start" page.  Use your Colby login as explained in the email from Randy Downer.  NEW JOB menu-Create a new job will bring you to a building window.  Using the BUILD menu, build ethanol. Use the CLEANUP menu to add hydrogens and optimize the geometry. You can rotate the molecule by using the arrow icons in the window.  CHOOSE ENGINE from the left menu: Gaussian.  JOB OPTION from the left menu will set up the calculations needed for the Raman/IR vibrational modes:  Calculation=optimize and vib. freq, Theory=Hartree Fock, Basis Set+Routine 6-31G(d), Charge=0, Multiplicity=singlet. (The advanced tab will allow you to specify water as the solvent if you would like to compare the calculations. The calculations with a solvent take longer.) SUBMIT JOB from the left menu.  When finished, double click on the blue molecular formula and select RAW OUTPUT from the left menu.  Scroll through the data to find the IR/Raman results.  Use the data viewer window to see the IR spectrum and the animated vibrational modes.  The original molecule in the viewer will now have the atoms numbered to match the raw output information.  This will allow you to match the vibrational frequencies and the type of vibration with the correct bond so that you can make predictions about the IR and Raman activity of ethanol. Please log out of the job manager window when you are finished.  You can access this applet from anywhere on campus.  (NOTE: multiplying the Hartree Fock values by 0.9 will adjust the frequency values for "harmonization" issues.)

You will be combining the IR and Raman data in an ACS formatted lab report.  To help you in the proper formatting of this paper, please visit the information to authors on the web, the ACS style guide in the PQ lab and the example of Prof. King's publication (found in the “formal paper” blue binder in the lab).  Your standard addition calculations and the statistical analysis of [EtOH] in vodka should be presented in excel using the pre-lab template. Please hand in this information with the ACS format (supplemental info).  Be sure to refer to the questions included in this handout that must be addressed in your report. Be careful, as outlined in the ACS format, not all of the raw data should be included in the ACS report!  This report will count 25% of the lab grade.

Note: You have TWO weeks to complete this laboratory. Focus on measurements using ONE of the methods each week. We will start with FTIR.

Pre-lab Sample Preparation: Please refer to the laboratory instructions below for sample preparation. The same solutions will be used for both the IR and Raman measurements. Therefore, when determining total sample volume, be prepared to perform each measurement on each instrument AT LEAST THREE (3) times. The FTIR requires two drops of sample, while the Raman will use a 3 mL cuvette. Carefully prepare all standard addition samples being sure to use the most appropriate measurement devices volume and/or mass accuracy.

HINT: To make your calculations easier in the end, consider having the same total volume for each standard addition sample – this means you will need to add varying amounts of absolute ethanol and water to each sample.  The sum of the volumes for the absolute ethanol, water and (constant volume of) vodka sample should be the same for all samples (please see the instructor if you have questions). A 2-fold dilution of your unknown (vodka) in each standard addition sample is recommended.

If you are prepared and have your calculations ready at the start of lab, sample preparation should take less than one hr. (Refer to Harris, section 5-3)

References:

• Journal of Chemical Education 78(2001)1221.

• Journal of Chemical Education 80(2003)672.

• For information on the standard additions method and how to prepare samples, see Harris, pp. 106-108, Section 5-3.

• For information on the attenuated total reflectance (ATR) cell used in the FTIR measurements, see Harris, pp. 463-465.

• For comparison of IR and Raman methods, see Skoog pp 481-497.

NOTE: These are very useful references and may be used in your report. However, as described in the lab notes, you will need to find at least on “new” reference for your report.

Goals:

1. To prepare standard addition samples for quantitative measurements of an analyte in a complex matrix.

2. To perform quantitative measurements of these samples using the attenuated total reflectance (ATR) cell on the FTIR.

3. To perform quantitative measurements of the same samples using the Raman instrument.

4. To compare (quantitatively whenever possible!) these two methods in their ability to quantitate ethanol in vodka samples.

5. To calculate the theoretical vibrational energies of ethanol (using normal mode analysis in WebMo-Gaussian) and compare them to the observed IR and Raman frequencies.

6. Based on the theoretical calculations for water, compare the IR and Raman techniques for analysis of aqueous samples.

Laboratory Instructions:

A. Sample Preparation: You will be preparing your own samples.

A vodka sample is provided. Assuming that the concentration of ethanol in vodka is approximately 40%, prepare a series of AT LEAST four standard addition samples suitable for analysis of ethanol in your sample. (PRE-LAB CALCULATIONS!!)

Pure ethanol (100%, 200-proof, dehydrated) should be used for your additions.

E-pure water should be used to achieve final volume.

B. Materials Available:

Unknown solution: Vodka sample (~40% ethanol by volume)

Pure ethanol (100%, 200-proof)

18 MΩ water

Volumetric glass pipettes and bulbs (in 1, 2, 3, 4, 5, 10, 15, and 25 mL sizes)

Volumetric flasks with caps (in 10, 25, 50, and 100 mL sizes)

Plastic 50-ml tubes for sample storage (Please do not store samples in volumetric flasks!)

ATR Cell for FTIR: ~2 drops of sample needed per measurement

1-cm Quartz cuvette for Raman: ~3 mL of sample needed per measurement

C. FTIR Measurements:

Ethanol signals throughout the spectrum will be observed but quantitative measurements will be made at 1044 cm-1. (Why??) Solutions containing water absorb too strongly in the IR to use conventional sample cells. Instead we will use an ATR cell. The cell uses diamond zinc selenide crystal surrounded by your sample. The IR light beam is focused so that it bounces off of the crystal-sample interface once as it propagates through the cell. If the light at a given wavelength is absorbed by the sample, the amount of reflected light will be reduced or attenuated, and thus the name of the technique: Attenuated Total Reflectance (ATR).

Your instructor will demonstrate the operation of the Perkin Elmer Spectrum One FTIR with the ATR cell. (Simplified instructions are included as an appendix at the end of this handout). Collect the spectrum, label all peaks of interest and print the spectral data of water, vodka and absolute ethanol against the instrumental background signal. Using water as a blank sample, collect the spectrum for each sample on the PE Spectrum One FTIR spectrometer. Use the following FTIR settings for these measurements:

Scan Region 4000 to 400 cm-1, Resolution 4 cm-1, Scans 16

Collect the quantitative measurements for the signal at ~1044 cm -1 for each spectrum (signal intensity and area are recommended. Why?). Be sure to use the same wavenumber for each measurement. For absolute ethanol, be sure to note the wavenumber and intensity of EVERY signal originating from the sample (to compare to theoretical data for ethanol).

D. Raman Measurements:

Your instructor will demonstrate the layout and operation of the Raman instrument. (Simplified instructions are included beside the instrument.) Be sure to take notes on the layout and the instrument specifications. Calibrate the instrument with CCl4 or toluene to convert the pixel number of each signal to a wavenumber. The samples should be contained in a conventional 1-cm optical cell. Measure the Raman signal of water against the instrumental blank. Then measure vodka, absolute ethanol and each sample using water as the sample blank. Determine and record the wavenumber and intensity of the maximum signal due to ethanol. For absolute ethanol, be sure to note the wavenumber and intensity of EVERY signal originating from the ethanol (to compare to theoretical data for ethanol).

NOTE: Be sure to tabulate all data in your notebook, as there may not be a printer connected to this instrument!

E. Vibrational Mode Calculations:

Calculate the normal modes for ethanol using WebMo – Gaussian. Record the frequencies, relative intensities, and origins (type of vibration) of the IR and Raman active normal modes for this molecule. Please use this information to help you identify analyte signals and to make comparisons described in the next section.

Questions to be addressed in the (appropriate section) ACS report:

1. A description of the vodka sample you analyzed (name, reported alcohol content, etc.).

2. A properly formatted table of your results (and stats).

3. For each instrumental method of analysis (IR and Raman): indicate the frequency (wavenumber) used to measure the ethanol concentration of the samples.

4. [Ethanol] in the vodka sample (report the value in percent volume with appropriate uncertainties).

5. Compare the measurements from the two spectroscopic methods. Be as quantitative as possible. Which is more accurate? Which is more precise? Which is more sensitive? Wavenumber range? How do the methods compare in terms of time per sample and technical expertise of the user? Etc!!

6. Use your answers to the previous questions to justify which method you believe to be better for this application.

7. Create a table to tabulate the normal mode calculations, the vibrations (identify the motion and bond) most likely observed in the modes and the experimental modes. From the results of the calculations of ethanol normal modes, describe the bond and the IR and Raman vibrations most likely responsible for the IR and Raman signals in your samples.

8. For as many normal modes as possible, plot these theoretical frequencies vs. the experimentally observed frequencies for both IR and Raman. Briefly discuss these plots. Are all Raman peaks IR active and visa versa? Are the observed intensities of peaks as expected from theory? Please explain your observations.

9. Compare the IR and Raman spectra of water. Compare the IR and Raman techniques for analysis of aqueous samples.

10. (Supplemental information) In a single, well-documented EXCEL table, please tabulate your sample preparation data. Be sure to include all volumes used, each sample/solvent/solution, etc.

11. (Supplemental Information) The Excel standard addition spreadsheet for both Raman and IR used to calculate [ethanol].

12. Labeled IR and Raman spectra for the absolute ethanol sample.

APPENDIX: Directions for the use of the PE Spectrum One FTIR

- Open the Spectrum One shortcut icon on the desktop.

- Select Spectrum One in the instrument selection window  OK.

- You will be prompted to do a background (instrumental) scan.

- Select OK to run a sample. Check the instrumental parameters by selecting InstrumentScan. Use the toggle tabs to select:

Name (of file)

Scan 4000 to 400 cm-1

Scan Sample, Units A for absorbance, Scan = 8 scans, Resolution = 16

- Click on the APPLY button on the upper left of the window. When the button changes to   SCAN, press to scan. WATER should be your first sample.

- The data should be visible in the display window. When all eight scans are completed, the data will be displayed in the spectral window. (If it did not load, rescan.)

NOTE: To avoid rapid evaporation of the ethanol samples, set and apply the parameters, load the sample, then press scan.

- Use the Process  View optimize to improve the display of the spectrum.

- Using the window icons, adjust the Y scale so that the maximum number of peaks are “on scale”.

- Using the window icons, labels all of the peaks present in the spectrum.

- Using the window icon (ABC), add text to title the spectrum.

- Use File  Print or the print icon to print the spectrum. It will print to the printer connected to the instrument.

- Now run water as a sample blank to remove the water peaks from future spectral data. Use the window icon to use water as a background sample. Rerun the water sample as a sample. What should you see if the instrument was properly “blanked” with water?

- Continue to run vodka, absolute ethanol and each standard addition sample using Instrument- Scan.

- Label the title and peaks for each sample. Print the spectrum.

- To analyze the ~1044 cm-1 peak for area and height, select Process  peak/height. Drag the green cursor to the middle of the peak. This will be the wavenumber that should be used for all of the calculations. Choose height from the menu on the left. Record the height. Choose area from the menu on the left. Drag the two points of the baseline cursor too define the peak. The better the baseline, the better the two area calculations will match. Record the data or print the data window that will appear when you close this process window.

NOTE: The filename of choice must be highlighted in the lower left of the spectrum window for the data to be analyzed. You can get rid of unwanted spectral overlay by highlighting the unwanted filename and using delete.